# Birzeit University

# Faculty of Engineering and Technology

# ENCS5337: Chip Design Verification

# Homework #5: Randomized Verification of a Packet Filter

## 1. Objective

Verify a provided RTL module (bzu_pkt_filter using SystemVerilog randomization. Students must generate randomized packet transactions, apply them to the DUT, and verify correctness.

## 2. DUT Interface

Inputs:
clk,

rst_n,

valid_in,

dst_mac[47:0],

src_mac[47:0],

eth_type[15:0],

length[10:0],

has_vlan,

vlan_id[3:0]

Outputs:
valid_out,

accept,

drop_reason[2:0]

DUT RTL code: https://www.edaplayground.com/x/9txc

### Operation Overview

The bzu_pkt_filter module evaluates incoming packets and determines whether they should be accepted or dropped. Packets are sampled when valid_in is asserted and results are produced one clock cycle later.

#### Drop Conditions

1. Invalid Length: length < 64 or length > 1500

2. Broadcast Not Allowed: dst_mac = FF:FF:FF:FF:FF:FF when broadcast_enable = 0

3. Multicast Not Allowed: dst_mac[40] = 1 when multicast_enable = 0

4. Invalid Ethernet Type: not IPv4 (0x0800), ARP (0x0806), or IPv6 (0x86DD)

5. Invalid VLAN: has_vlan = 1 and vlan_id outside allowed range

#### Acceptance Condition

A packet is accepted only if all checks pass. In that case, accept = 1 and drop_reason = 0.

#### Priority of Checks

Checks are evaluated in the following order:

1. Length

2. Broadcast

3. Multicast

4. Ethernet Type

5. VLAN

#### Outputs

valid_out indicates output is valid.

accept indicates packet acceptance.

drop_reason indicates the reason for dropping the packet.

## 3. Timing Behavior

Inputs are sampled on rising edge when valid_in = 1.
Outputs are produced one clock cycle later.
valid_out indicates output is valid.

## 4. Drop Reason Encoding

0: ACCEPT
1: INVALID LENGTH
2: BROADCAST DISABLED
3: MULTICAST DISABLED
4: INVALID ETH_TYPE
5: INVALID VLAN

## 5. Packet Specification

Fields: dst_mac, src_mac, eth_type, length, has_vlan, vlan_id, payload (dynamic array), pkt_id (randc).

## 6. Packet Validity Rules

length 64–1500, valid eth_type (IPv4, ARP, IPv6), src_mac not broadcast/multicast, VLAN rules enforced, payload size equals length.

## 7. Testbench Requirements

Generate packets, drive DUT, capture outputs, compare expected vs actual behavior.

## 8. Required Tests

# Test 1: Random Valid Packets

Generate at least 50 packets using only valid constraints. Drive them to the DUT and verify that all packets are accepted. Print inputs and outputs and confirm accept=1 and drop_reason=0.

# Test 2: Directed Protocol Testing

Generate packets where eth_type is fixed to IPv4 (0x0800) and then ARP (0x0806). Drive packets and verify DUT accepts them. Ensure other fields remain randomized.

# Test 3: Boundary Length Testing

Generate packets with length = 64 and length = 1500 and verify they are accepted. Then generate packets with length < 64 and > 1500 and verify they are dropped with correct reason.

# Test 4: VLAN Behavior

Generate packets with has_vlan = 1 and valid vlan_id (1–10) and verify acceptance. Then generate packets with invalid vlan_id and verify DUT drops them with VLAN error.

# Test 5: Broadcast and Multicast

Generate packets with broadcast destination address (all bits 1) and multicast addresses (dst_mac[40]=1). Verify DUT behavior based on enable signals.

| Dst Mac | cfg | Expected Output |
| --- | --- | --- |
| FF:FF:FF:FF:FF:FF | broadcast_enable = 1 | Packet is ACCEPTED |
| FF:FF:FF:FF:FF:FF | broadcast_enable = 0 | Packet is DROPPED with DROP_BROADCAST |

| Dst Mac | cfg | Expected Output |
| --- | --- | --- |
| dst_mac[40] = 1 | multicast_enable = 1 | Packet is ACCEPTED |
| dst_mac[40] = 0 | multicast_enable = 0 | Packet is DROPPED with DROP_MULTICAST |

# Test 6: Weighted Traffic

Use distribution to generate more IPv4 packets than ARP and IPv6. Generate at least 100 packets and count occurrences of each type.

# Test 7: Constraint Disable

Disable a constraint such as length constraint. Generate invalid packets and verify DUT drops them.

# Test 8: Fixed Field

Disable randomization for src_mac, assign a fixed value, and verify it remains unchanged across multiple packets.

# Test 9: Partial Randomization

Randomize only selected fields such as length and eth_type while keeping others constant. Show difference before and after randomization.

# Test 10: Inline Constraint Override

Use inline constraints to restrict packet fields (e.g., smaller length range). Verify that generated packets follow the temporary constraint.

# Test 11: Constraint Validation

Generate a valid packet, manually modify it to become invalid, and verify constraint checking detects the issue.

# Test 12: Payload Constraints

Ensure payload size equals packet length. Constrain payload values and print part of payload for verification.

# Test 13: Cyclic Packet ID

Use cyclic randomization for packet ID and show that values do not repeat until all possibilities are used.

# Test 14: Solve Order

Apply solve-before between variables and generate many packets. Observe and compare distribution changes.

# Test 15: Scenario-Based Sequence

Generate a sequence of packets such that no two consecutive packets have the same eth_type. Print sequence and verify the rule is satisfied.

## Deliverables:

- Packet class: including different required constraints

- Tests classes. Put each test in a separate file

- Testbench with Clk frequency is 2GHz

- Tasks:

  - Generator (user transmit method). User to Create packet and transmit it

  - Agent: pop packet and pass it to both driver and cheker. User queues

  - Driver: gets a driver and write it to the DUT interface. It uses clocking block

  - Monitor: collects responses (using interface and clocking blocks) and passes them to checker

  - Checker: compares input with output

- End of test report

- Logs per test